| Student Name | <<enter Full Name >> |
| --- | --- |
| SRN No | <<enter SRN No >> |
| Roll No | <<Enter Roll No>> |
| Program | <<AIDS / Computer Engg.>> |
| Year | Third Year |
| Division | <<Enter division>> |
| Subject | Computer Network Laboratory (BTECCE23506) |
| Assignment No | Eight |

Assignment Number - 07

Title : Configuration of router for implementation of EIGRP protocol.

Problem Statement : Configure Routers for EIGRP implementation

Theory :

EIGRP is a Cisco proprietary routing protocol. It is one of the most popular and widely used routing protocols. It is a complex routing protocol, but it provides many benefits and supports all sizes and types of networks.

Configuring EIGRP on routers

EIGRP configuration is straightforward. It is a two-step process. In the first step, we enable EIGRP. In the second step, we provide the required details to the EIGRP. Let’s discuss both steps in detail.

Enabling EIGRP

To enable EIGRP, we use the following command in global configuration mode.

Router(config)# router eigrp AS_number

The above command enables EIGRP and puts us in EIGRP configuration mode. It needs only one parameter, an AS (Autonomous System) number. EIGRP uses an AS number to create a group of routers that can share routing information. You can choose any number from the range 1 to 65535 as an AS number. After selecting a number, you have to use the same number on all routers. If two routers belong to different AS, they will not share routing information.

### K-Values and EIGRP Metrics

K-Values are the most important part of EIGRP. Usually newbies take K Values as EIGRP metric components. K Values are not the metric components in themself. They are only the place holder or influencer for actual metric components in metric calculation formula. So when we enable or disable a K value, actually we enable or disable its associate metric component.

EIGRP uses four components out of five to calculate the routing metric.

| K Value | Component | Description |
| --- | --- | --- |
| K1 | Bandwidth | Lowest bandwidth of route |
| K2 | Load | Worst load on route based on packet rate |
| K3 | Delay | Cumulative interface delay of route |
| K4 | Reliability | Worst reliability of route based on keep alive |
| K5 | MTU | Smallest MTU in path [Not used in route calculation] |

## EIGRP Metric Calculation Formula

EIGRP uses following formula to produce a single 32 bit metric:-

At first glance this formula looks like a complicated equation. But it is not as difficult as it sound. Let’s make it easier.

As we know MTU (K5) is not actively participate in formula. So set its value to Zero. When K5 is equal to 0 then [K5/ (K4 + reliability)] is defined to be 1.

By default EIGRP does not use dynamic values in metric. This will disable two more components; load (K2) and reliability (K4).

Now only two static values remain in formula.

Use of default constants [K1 (Enabled), K2 (Disabled), K3 (Enabled), K4 (Disabled), K5 (Disabled not used)] reduce our formula to:-

Metric = (BandwidthE + DelayE)*256.

Cisco uses following configuration values for Bandwidth and delay

BandwidthE = 107/ least bandwidth of route [Lowest bandwidth from all interfaces between source and destination. Use interface default bandwidth wherever bandwidth is not set through the bandwidth command]

ValueE = cumulative delay of route [Sum of all outgoing interface’s delay. Use interface default delay, if not set through the delay command]

Putting these configuration values will make formula to look like this

Before we move further, let me explain why EIGRP keeps dynamic values disable by default.

Dynamic values change over the time. Enabling dynamic values will force EIGRP routers to calculate metric all the time and send updates each other just because the load or reliability of an interface has changed. This will create serious performance issue. To avoid such a situation EIGRP only enables static values for metric calculation.

If we only enable static values for metric calculation, EIGRP will not recalculate the metric unless it changed. Static values change only when a physical change occurred in network such as an interface is down or router is dead. This will keep EIGRP nice and clean.

Conclusion : <<Write Conclusion in your own words. Write about what you learn from OSPF protocol implementation >>

EIGRP CONFIGURATION

Router R0 Configuration

Router(config)#interface fastEthernet 0/0

Router(config-if)#ip address 10.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fastEthernet 0/1

Router(config-if)#ip address 192.168.1.1 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router#show controllers serial 0/0/0

Interface Serial0/0/0

Hardware is PowerQUICC MPC860

DCE V.35, clock rate 2000000

[Output omitted]

Router# configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface serial 0/0/0

Router(config-if)#ip address 192.168.1.245 255.255.255.252

Router(config-if)#clock rate 64000

Router(config-if)#bandwidth 64

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router eigrp 20

Router(config-router)#network 10.0.0.0 0.0.0.255

Router(config-router)#network 192.168.1.244 0.0.0.3

Router(config-router)#network 192.168.1.0 0.0.0.3

Router(config-router)#

Router#show ip route eigrp

D    20.0.0.0/8 [90/35840] via 192.168.1.2, 00:31:21, FastEthernet0/1

192.168.1.0/24 is variably subnetted, 7 subnets, 2 masks

D       192.168.1.0/24 is a summary, 00:31:22, Null0

D       192.168.1.4/30 [90/30720] via 192.168.1.2, 00:31:21, FastEthernet0/1

D       192.168.1.8/30 [90/33280] via 192.168.1.2, 00:31:21, FastEthernet0/1

D       192.168.1.248/30 [90/2689536] via 192.168.1.2, 00:31:16, FastEthernet0/1

D       192.168.1.252/30 [90/2177536] via 192.168.1.2, 00:31:21, FastEthernet0/1

Router#

Router#show interface serial0/0/0

Serial0/0/0 is up, line protocol is up (connected)

Hardware is HD64570

Internet address is 192.168.1.245/30

MTU 1500 bytes, BW 64 Kbit, DLY 20000 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation HDLC, loopback not set, keepalive set (10 sec)

Last input never, output never, output hang never

Last clearing of "show interface" counters never

Input queue: 0/75/0 (size/max/drops); Total output drops: 0

Queueing strategy: weighted fair

Output queue: 0/1000/64/0 (size/max total/threshold/drops)

Conversations  0/0/256 (active/max active/max total)

Reserved Conversations 0/0 (allocated/max allocated)

Available Bandwidth 48 kilobits/sec

5 minute input rate 102 bits/sec, 0 packets/sec

5 minute output rate 104 bits/sec, 0 packets/sec

426 packets input, 25543 bytes, 0 no buffer

Received 0 broadcasts, 0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored, 0 abort

423 packets output, 25289 bytes, 0 underruns

0 output errors, 0 collisions, 1 interface resets

0 output buffer failures, 0 output buffers swapped out

Router R1 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface serial 0/0/0

Router(config-if)#ip address 192.168.1.246 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/0/1

Router(config-if)#ip address 192.168.1.249 255.255.255.252

Router(config-if)#clock rate 64000

Router(config-if)#bandwidth 64

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router eigrp 20

Router(config-router)#network 192.168.1.244 0.0.0.3

Router(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 20: Neighbor 192.168.1.245 (Serial0/0/0) is up: new adjacency

Router(config-router)#network 192.168.1.248 0.0.0.3

Router(config-router)#

Router R2 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface serial 0/0/1

Router(config-if)#ip address 192.168.1.250 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface serial 0/0/0

Router(config-if)#ip address 192.168.1.253 255.255.255.252

Router(config-if)#clock rate 64000

Router(config-if)#bandwidth 64

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router eigrp 20

Router(config-router)#network 192.168.1.248 0.0.0.3

Router(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 20: Neighbor 192.168.1.249 (Serial0/0/1) is up: new adjacency

Router(config-router)#network 192.168.1.252 0.0.0.3

Router(config-router)#

Router R3 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface fastEthernet 0/0

Router(config-if)#ip address 192.168.1.6 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fastEthernet 0/1

Router(config-if)# ip address 192.168.1.9 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#router eigrp 20

Router(config-router)#network 192.168.1.8 255.255.255.252

Router(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 20: Neighbor 192.168.1.10 (FastEthernet0/1) is up: new adjacency

Router(config-router)#network 192.168.1.4 255.255.255.252

Router(config-router)#

Router R4 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface fastEthernet 0/0

Router(config-if)#ip address 192.168.1.5 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fastEthernet 0/1

Router(config-if)# ip address 192.168.1.2 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#router eigrp 20

Router(config-router)#network 192.168.1.4 255.255.255.252

Router(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 20: Neighbor 192.168.1.6 (FastEthernet0/0) is up: new adjacency

Router(config-router)#network 192.168.1.0 255.255.255.252

Router(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 20: Neighbor 192.168.1.1 (FastEthernet0/1) is up: new adjacency

Router(config-router)#

Router R5 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface fastEthernet 0/0

Router(config-if)#ip address 20.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface fastEthernet 0/1

Router(config-if)#ip address 192.168.1.10 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#

Router(config)#interface serial 0/0/0

Router(config-if)#ip address 192.168.1.254 255.255.255.252

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#router eigrp 20

Router(config-router)#network 20.0.0.0 255.0.0.0

Router(config-router)#network 192.168.1.252 255.255.255.252

Router(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 20: Neighbor 192.168.1.253 (Serial0/0/0) is up: new adjacency

Router(config-router)#network 192.168.1.8 255.255.255.252

Router(config-router)#