| Student Name | <<enter Full Name >> |
| --- | --- |
| SRN No | <<enter SRN No >> |
| Roll No | <<Enter Roll No>> |
| Program | <<AIDS / Computer Engg.>> |
| Year | Third Year |
| Division | <<Enter division>> |
| Subject | Computer Network Laboratory (BTECCE21506) |
| Assignment No | Four |

Assignment Number - 04

Title : Configuration of router by using router commands and implementation of static routing

Problem Statement Using a Network Simulator (e.g. packet tracer) Configure routers for static routing

Theory :

Router – Router is a network device that allows you to direct data traffic to an  appropriate destination. Router maintain routing table that contain IP addresses of computers over the network. A router has different components that enable proper functioning.

Cisco IOS supports various command modes, among those followings are the main command modes.

•  User EXEC Mode

•  Privileged EXEC Mode

•  Global Configuration Mode

•  Interface Configuration Mode

•  Sub Interface Configuration Mode

•  Setup Mode

| Mode | Prompt | Command to enter | Command to exit |
| --- | --- | --- | --- |
| User EXEC | Router > | Default mode after booting. Login with password, if configured. | Use exit command |
| Privileged EXEC | Router # | Use enable command from user exec mode | Use exit command |
| Global | Router(config)# | Use configure terminal command from privileged exec mode | Use exit command |
| Configuration |  |  |  |
| Interface | Router(config-if)# | Use interface | Use exit command to return in global configuration mode |
| Configuration |  | type number command from global configuration mode |  |
| Sub-Interface | Router(config-subif) | Use interface type sub interface numbercommand from global configuration mode or interface configure mode | Use exit to return previous mode. Use end command to return in privileged exec mode. |
| Configuration |  |  |  |

Some important router Command

| Command |  |  | Description |
| --- | --- | --- | --- |
| Router(config)#interface serial 0/0/0 |  |  | Enter into serial interface 0/0/0 configuration mode |
| Router(config-if)#description | Connected | to | Optional command. It set description on |
| Interface |  |  | interface that is locally significant |
| Router(config-if)#ip address 10.0.0.1 |  |  | Assigns address and subnet mask to Interface |
| 255.0.0.0 |  |  |  |
| Router(config-if)#clock rate 64000 |  |  | DCE side only command. Assigns a clock rate for the interface |
| Router(config-if)#bandwidth 64 |  |  | DCE side only command. Set bandwidth for the interface. |
| Router(config-if)#no shutdown |  |  | Turns interface on |

Configuration of  static  Routing

Code

Basic Router Configuration – Static Routing

Router0 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface Serial0/1/0

Router(config-if)#ip address 20.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/1/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/1/0, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/0/0

Router(config-if)#ip address 30.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/0/0, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/0/1

Router(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/0, changed state to up

ip address 50.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/0/1, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/1/1

Router(config-if)#ip address 70.0.0.1 255.0.0.0

Router(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/0/1, changed state to up

no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface Serial0/1/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Serial0/1/1, changed state to up

Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1

Router(config)#ip route 80.0.0.0 255.0.0.0 70.0.0.2

Router(config)#ip route 40.0.0.0 255.0.0.0 30.0.0.2

Router(config)#ip route 60.0.0.0 255.0.0.0 50.0.0.2

Router#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

S    10.0.0.0/8 [1/0] via 20.0.0.1

C    20.0.0.0/8 is directly connected, Serial0/1/0

C    30.0.0.0/8 is directly connected, Serial0/0/0

S    40.0.0.0/8 [1/0] via 30.0.0.2

C    50.0.0.0/8 is directly connected, Serial0/0/1

S    60.0.0.0/8 [1/0] via 50.0.0.2

C    70.0.0.0/8 is directly connected, Serial0/1/1

S    80.0.0.0/8 [1/0] via 70.0.0.2

Router#

Router1 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 80.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/0/0

Router(config-if)#ip address 70.0.0.2 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

Router(config)#ip route 10.0.0.0 255.0.0.0 70.0.0.1

Router(config)#ip route 20.0.0.0 255.0.0.0 70.0.0.1

Router(config)#ip route 30.0.0.0 255.0.0.0 70.0.0.1

Router(config)#ip route 40.0.0.0 255.0.0.0 70.0.0.1

Router(config)#ip route 50.0.0.0 255.0.0.0 70.0.0.1

Router(config)#ip route 60.0.0.0 255.0.0.0 70.0.0.1

Router2 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 60.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/0/0

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)#interface Serial0/0/0

Router(config-if)#ip address 50.0.0.2 255.0.0.0

Router(config-if)#

Router(config)#ip route 10.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 20.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 30.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 40.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 70.0.0.0 255.0.0.0 50.0.0.1

Router(config)#ip route 80.0.0.0 255.0.0.0 50.0.0.1

Router(config)#

Router3 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 10.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/0/0

Router(config-if)#ip address 20.0.0.1 255.0.0.0

Router(config-if)#clock rate 64000

Router(config-if)#no shutdown

Router(config-if)#

Router(config)#ip route 30.0.0.0 255.0.0.0 20.0.0.2

Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2

Router(config)#ip route 50.0.0.0 255.0.0.0 20.0.0.2

Router(config)#ip route 60.0.0.0 255.0.0.0 20.0.0.2

Router(config)#ip route 70.0.0.0 255.0.0.0 20.0.0.2

Router(config)#ip route 80.0.0.0 255.0.0.0 20.0.0.2

Router(config)#

Router#

%SYS-5-CONFIG_I: Configured from console by console

Router#

Router#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

C    10.0.0.0/8 is directly connected, FastEthernet0/0

C    20.0.0.0/8 is directly connected, Serial0/0/0

S    30.0.0.0/8 [1/0] via 20.0.0.2

S    40.0.0.0/8 [1/0] via 20.0.0.2

S    50.0.0.0/8 [1/0] via 20.0.0.2

S    60.0.0.0/8 [1/0] via 20.0.0.2

S    70.0.0.0/8 [1/0] via 20.0.0.2

S    80.0.0.0/8 [1/0] via 20.0.0.2

Router4 Configuration

Router>enable

Router#configure terminal

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#interface FastEthernet0/0

Router(config-if)#ip address 40.0.0.1 255.0.0.0

Router(config-if)#no shutdown

Router(config-if)#

%LINK-5-CHANGED: Interface FastEthernet0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface FastEthernet0/0, changed state to up

Router(config-if)#exit

Router(config)#interface Serial0/0/0

Router(config-if)#ip address 30.0.0.2 255.0.0.0

Router(config-if)#clock rate 64000

This command applies only to DCE interfaces

Router(config-if)#no shutdown

Router(config-if)#do write

Building configuration...

[OK]

Router(config-if)#

Router#config t

Enter configuration commands, one per line.  End with CNTL/Z.

Router(config)#ip route 10.0.0.0 255.0.0.0 30.0.0.1

Router(config)#ip route 20.0.0.0 255.0.0.0 30.0.0.1

Router(config)#ip route 50.0.0.0 255.0.0.0 30.0.0.1

Router(config)#ip route 60.0.0.0 255.0.0.0 30.0.0.1

Router(config)#ip route 70.0.0.0 255.0.0.0 30.0.0.1

Router(config)#ip route 80.0.0.0 255.0.0.0 30.0.0.1

Conclusion : <<Write Conclusion in your own words. Write about what you learn from assignment>>